#include <Servo.h>

#define TRIGGER_PIN  9 // Arduino pin connected to the trigger pin of ultrasonic sensor

#define ECHO_PIN     10 // Arduino pin connected to the echo pin of ultrasonic sensor

#define SERVO_PIN    11 // Arduino pin connected to the signal pin of servo motor

#define MAX_DISTANCE 20 // Maximum distance threshold for triggering servo (in centimeters)

Servo servo;

void setup() {

Serial.begin(9600);

pinMode(TRIGGER_PIN, OUTPUT);

pinMode(ECHO_PIN, INPUT);

servo.attach(SERVO_PIN);

}

void loop() {

long duration, distance;

digitalWrite(TRIGGER_PIN, LOW);

delayMicroseconds(2);

digitalWrite(TRIGGER_PIN, HIGH);

delayMicroseconds(10);

digitalWrite(TRIGGER_PIN, LOW);

duration = pulseIn(ECHO_PIN, HIGH);

distance = (duration / 2) / 29.1; // Calculate distance in centimeters

if (distance <= MAX_DISTANCE) {

// If object is within range, open the lid

servo.write(90); // 90 degrees position (adjust as needed)

delay(1000); // Wait for 1 second

} else {

// If no object is detected, close the lid

servo.write(0); // 0 degrees position (adjust as needed)

}

Serial.print("Distance: ");

Serial.print(distance);

Serial.println(" cm");

delay(1000); // Wait for 1 second before taking the next reading

}